Integration Note

| Manufacturer: | Rollease Acmeda |
| --- | --- |
| Model Number(s): | Automate Pulse 2 |
| g! Core Module Version: | 8.3 |
| Driver Developer: | annex4 |
| Document Revision Date: | 05/30/2019 |

Overview & Supported Features

This driver is a Lighting Interface Controller for the Rollease Acmeda Automate Pulse 2 Hub. It allows IP control over Rollease shades from ELAN.

The following options are supported by this driver:

- Dimmer, Shade, and Louvre lighting devices.

- Realtime feedback of shade positions updated to ELAN Core.

- Open, and Ramp Up commands.

- Close, and Ramp Down commands.

- Stop to halt the movement of rotation or positioning.

- Rotation accurate to 1.8% increments.

- Position accurate to 1% increments.

Any feature not specifically noted as supported should be assumed to be unsupported.

Automate Pulse Hub Configuration

- The hub will need to be installed and configured with the Automate Pulse application. All shades should be configured with the Automate Pulse application before starting the installation of the ELAN driver.  Please refer to the Automate Pulse application for step by step instructions on how to add shades to your hub.

g! Configuration

- As with all third-party drivers these do not reside in the typical listing of drivers from ELAN. To add this driver to an ELAN system:

- Download the .EDRVC file associated with this document if you do not already have it.

- Place the .EDRVC file in a location on your computer.

  - I would recommend \User\<User>\Documents\Elan\Drivers.

- In the g! Configurator navigate to the Lighting Interfaces item under Lighting.

- Right click Lighting Interfaces and select “Add New Lighting Interface…”.

- Click the “Search Folder…” button at bottom of the new window that opened.

- Navigate to the location you saved the .EDRVC file to and select it.

- Select “Rollease Acmeda Automate v2” from the list and click “OK”.

- A prompt will appear saying there will be no support from ELAN, click “OK”.

- The driver should now be visible under the Lighting Interfaces header.

Installation Process

It is recommended that you follow the below installation process in order to proper functionality with the driver.

- Select the Interface Type you want to use in Elan to control the shades.

  - You can do this by setting the configuration option “Interface Type” to “Dimmer”, “Shade”, or “Both”.

- Enter the IP address of the Automate Pulse 2 Hub in the “IP Address” field.

- Click “Discover Devices” at the bottom of the lighting interface window.

  - Please wait a few seconds for the driver to find all the shades on the hub.

- At this point your shades should be created under the lighting interface.

- [Optional] If you want to control the rotation of the shade you can add a Louvre device

  - Select Lighting Devices (None) or any of the shades that have been created.

  - Select “Add New Device”.

  - Select “Louvre Tilt”.

  - Enter the Address of the shade in the “Shade Address” configuration line of the new device.

  - Enter “LOUVRE” (without quotes) into the Device Type field.

- Bind the devices to user interface components in the custom pages.

g! Configuration Details

The following table provides settings used in Configurator.  In the table below:

- “<User Defined>”, etc.			Type in the desired name for the item.

- “<Auto Detect>”, etc.			The system will auto detect this variable.

- “<System>”, etc.				The setting has been automatically set by the system.

- “<Select>”					A dropdown with a list of options.

| Devices | Variable Name | Setting | Comments |
| --- | --- | --- | --- |
| Lighting Interface | Name | <User Defined> | Default “Rollease Acmeda Automate v2” |
|  | System # | <System> | Defined by the system to identify the device |
|  | Status Color Coding | <Select> | Enabled or Disabled |
|  | Status | <System> | The state of the driver, where green is good and red means there was an installation problem. |
|  | Driver Version | <System> | The version of the driver |
|  | Driver Vendor | <System> | Annex4 |
|  | Installed | <System> | The date and time the driver was installed |
|  | Device Type | <System> | The label identifying what type of device the driver is |
|  | Shades | <System> | The listing of shades discovered by the driver |
|  | Debug Mode | <Select> | Select the verbosity of the entries that should appear in the Driver Log |
|  | Interface Type | <Select> | Select the type of devices that will be created when discovery occurs |
|  | IP Address | <User Defined> | The IP address of the Pulse Hub |
|  | Port | <User Defined> | Defaults to 1487, this should not be changed unless the Hub itself has changed its IP address |
| Light Device | Name | <User Defined> | The name of the device |
|  | Location | <Select> | The location of the device in the project |
|  | System # | <System> | Defined by the system to identify the device |
|  | Device Type | <System> | The type of device |
|  | Hide Device From Scheduler | <Select> |  |
|  | Shade Address | <User Defined> | The address of the shade to control |
|  | Type | <User Defined> | SHADE, DIMMER, or LOUVER. |

Common Mistakes

- Entering the wrong IP address in the “IP Address” configuration line.

  - If you’re failing to discover devices double check this!

ELAN Developer Partner Information

- This ELAN driver was written and supported by:

- Feel free to contact us for feature requests, bug reports, or assistance with the driver.

- TheDesk@annex4.com